Not to exceed one page

ANNEXURE – V (A)

Title of the Invention: Novel Concrete Composition for Impact Energy Dissipating Crash Barriers.

Inventor(s):

| Name: Mr. Al-Amir Haruna Saleh | Name: Mr. Pratik Kanungo | Name: Dr. Anush Konayakanahalli Chandrappa | Name: Prof. Dinakar Pasla |
| --- | --- | --- | --- |
| Position: Post-Graduate student | Position: Research Scholar | Position: Associate Professor | Position: Professor |
| Department: School of Infrastructure | Department: School of Infrastructure | Department: School of Infrastructure | Department: School of Infrastructure |
| Phone: +234-8137781806 | Phone: +91-9668653751 | Phone: +91-6284654627 | Phone: +91-9556297825 |
| E-mail: 24SE06013@iitbbs.ac.in | E-mail: A23CE09018@iitbbs.ac.in | E-mail: akc@iitbbs.ac.in | E-mail: pdinakar@iitbbs.ac.in |

Brief Description of the Invention (not more than 100 words):

The invention relates to a novel structural lightweight concrete composition incorporating sintered fly ash aggregates, ground granulated blast-furnace slag, and hooked-end steel fibers for impact-resistant crash-barrier applications. The developed material exhibits reduced density, enhanced ductility, superior impact energy absorption, controlled deformation and reduced fragmentation while maintaining structural-grade strength. The invention further provides an energy-dissipating crash barrier system that reduces force transmission and occupant deceleration during vehicular collisions. The technology promotes the sustainable utilization of industrial by-products, reduces dependence on natural aggregates, and offers significant potential for transportation infrastructure, protective structures, and precast impact-resistant applications.

Novelty:

- First known application of steel fiber reinforced sintered fly ash lightweight concrete specifically for crash barrier applications.

- Novel combination of Sintered Fly Ash Aggregate (SFA), Hooked-End Steel Fibers, Ground Granulated Blast Furnace Slag (GGBS), for developing an impact-resistant, lightweight structural concrete system.

- Development of energy dissipating rigid barrier material exhibiting enhanced impact dissipating capacity, reduced fragmentation, improved post-cracking integrity, and reduced occupant force transmission.

- Simultaneous achievement of structural-grade strength, lightweight characteristics, superior energy absorption, and sustainability.

Inventiveness:

The invention overcomes the conventional perception that lightweight concrete is unsuitable for crash barrier applications due to its lower stiffness and impact resistance. The synergistic interaction among porous sintered fly ash aggregates, steel fiber crack-bridging mechanisms and matrix enhancement through GGBS results in unexpected improvements in impact energy absorption, ductility, post-cracking behaviour, and fragmentation resistance. Such combined performance cannot be reasonably predicted from existing technologies and therefore demonstrates significant inventive merit.

List Probable Users:

- Government Agencies

- Ministry of Road Transport and Highways (MoRTH)

- National Highways Authority of India (NHAI)

- State Public Works Departments (PWDs)

- National High Speed Rail Corporation Ltd.

- Metro Rail Corporations

- Airport Authority of India

- Industries

- Precast concrete manufacturers

- Crash barrier manufacturers

- Infrastructure construction companies

- Bridge construction agencies

- Defence infrastructure agencies

- Smart city development agencies

- Agencies involved in the generation of fly ash

- Patent search facilities used:-

The following patent databases and literature repositories were consulted:

- Google Patents

- Espacenet

- Indian Patent Advanced Search System (InPASS)

- Google Scholar

- Scopus

- ScienceDirect

- Web of Science

- Key words used before search (maximum 10):

- Crash Barrier Concrete

- Impact Resistant Concrete

- Lightweight Aggregate Concrete

- Steel Fibre Reinforced Concrete

- Sintered Fly Ash Aggregate

- Energy Absorbing Barrier

- Sustainable Crash Barrier

- Fibre Reinforced Lightweight Concrete

- Highway Safety Barrier

- Impact Dissipating Concrete

|  | 16/07/2026 | School of Infrastructure, IIT Bhubaneswar, Khordha, Odisha, 752050 |
| --- | --- | --- |
| Inventor’s signature | Date | Place |
| Anush K Chandrappa | 16/07/2026 | School of Infrastructure, IIT Bhubaneswar, Khordha, Odisha, 752050 |
| Inventor’s signature | Date | Place |
|  | 16/07/2026 | School of Infrastructure, IIT Bhubaneswar, Khordha, Odisha, 752050 |
| Inventor’s signature | Date | Place |
| Dinakar Pasla | 16/07/2026 | School of Infrastructure, IIT Bhubaneswar, Khordha, Odisha, 752050 |
| Inventor’s signature | Date | Place |

______________________________________________________________________________

Signature of the Inventor(s) with date